Digital Filters

Ying Sun

Digital filters

Finite impulse response (FIR) filter:  h[n] has a finite numbers of terms.

Infinite impulse response (IIR) filter:   h[n] has infinite numbers of terms.

Causal filter:  hn=0, for all n < 0.

Noncausal filter: hn≠0, for at least one  n < 0.

Stability

n=−∞∞hn<∞⇒ stable

FIR filters are always stable. IIR filters may or may not be stable.

Evaluating filter output by convolution

yn=m=−∞∞xn−mhm.=.m=−∞∞xmhn−m

Example 1.  The impulse response of a filter

is hn=e−nun. The input is a square pulse:

xn=1,aan=0,1,2,30,aaaotherwise.   Determine y[n].

yn.=.m=−∞∞xmhm−n.=.

0,aaaaaaaaaan<0m=0ne−n−m,aa0≤n≤3m=03e−n−m,aaaan>3.=.

0,aaaaaaaaaaaaaaaaaaaaaaaaan<01,aaaaaaaaaaaaaaaaaaaaaaaaan=01+e−1,aaaaaaaaaaaaaaaaaaaaan>11+e−1+e−2,aaaaaaaaaaaaqqaaan=21+e−1+e−2+e−3,aaaaaaaaaaaaan=3e−n+e−n−1+e−n−2+e−n−3,aaan>3

This is a low-pass filter. For the square pulse input, the output is a charge-discharge curve that shows reduction of the high-frequency contents.

Example 2. We implement a 3-point FIR low-pass filter to smooth the data according to:

yn=14xn+2xn−1+xn−2

If the sampling rate is 200 Hz, what is the cutoff frequency (3 dB roll-off) of this filter?

The impulse response as shown in figure is obtained  by setting xn=δn.

hn=14δn+2δn−1+δn−2

Take z-transform (ZT) of the filter equation above:

Yz=14Xz+2z−1Xz+z−2Xz.

The transfer function is given by

Hz.=.YzXz.=.141+2z−1+z−2.=.z−12z+2+z−12.=.z−121+z+z−12

The Fourier transform (FT) is the ZT evaluated on the unit circuit of the z-plane .

By replacing z=ejω, we have

Hejω.=.e−jω21+ejω+e−jω2

Using the Euler's formula cosω=ejω+e−jω2, we have

Hejω.=.e−jω21+cosω.

The magnitude and phase of Hejωare:

Hejω.=.121+cosω

∢Hejω.=.−ω

The system has a linear phase response, which represents a group phase delay. The linear phase is desirable because it does not introduce any phase distortion.

The DC gain of the filter is 1; Hej0=1+cos%02=1. The cutoff frequency fc is where the gain drops to 12 of the DC gain.

12.=.0.70794...;    20log100.70794.=.−3dB.

Let Hejωc.=.121+cosωc.=.12, we have

cosωc.=.2−1.=.1.414−1.=.0.414

ωc.=.cos−10.414.=.1.114 radians/s = 65.5o/s.

The sampling rate is 200 Hz, which corresponds to 2π or 360o. The cutoff frequency is:

fc.=.200×65.5o360o.=.36Hz.

Pole-zero plot: Hz.=.z2+2z+14z2.=.z+124z2. There are two zeros at -1 and two poles at the origin on the z-plane. The red line shows the vector from ejω to the pole. The green line shows the vector from ejω to the pole. The magnitude of Hejωis proportional to the green line squared over the red line squared, as ω goes from 0 to π.

Digital filters implemented with a microprocessor

Digital filters can be implemented in real time by using a microprocessor system as shown. In order to sample the data with a precise sampling interval, a timer is used to generate periodical interrupts. The interrupts are processed by the microprocessor unit (MPU). A running buffer is set up in the memory to store the present and previous input sample points. The filter should be implemented in the interrupt service routine with the following pseudocode:

x2 = x1;

x1 = x0;

x0 = read from A/D;

y0 = (x0 + x1 + x1 + x2) / 4;

write y0 to D/A;

Example 3. In the continuous time, the derivative is defined as dxtdt=limΔt→0xt−xt−ΔtΔt. In the

discrete time, it is not possible to make Δt→0. The smallest Δtis the sampling period T. Thus, the backward difference is a discrete-time approximation of the derivative. Without losing generality, we let T = 1 and add a scale factor of 1/2.

yn=xn−xn−12

Plot Hejωc and show the pole-zero plot.

The impulse response is:

hn=δn−δn−12

The transfer function is obtained by taking the ZT of h[n].

Hz.=.1−z−12.=.z−12z−0.=.z−12z12−z−122

The first-order system has one zero at 1 and one pole at the origin, as shown. To obtain the FT, let z=ejω. We have

Hejω.=.e−jω2ejω2−e−jω22.=.je−jω2ejω2−e−jω22j

Using the Euler's formula sinω=ejω−e−jω2j, we have

Hejω.=.je−jω2sinω2.=.e−jω2ejπ2sinω2.=.e−jω−π2sinω2.

The magnitude and phase of Hejωare:

Hejω.=.sinω2

∢Hejω.=.−ω−π2

Notice that for the continuous time, the LT of the derivative is s and the magnitude of the FT is Hω.=.ω, as shown by the orange line.  ω and sin(ω/2) are close for low frequencies,, but deviate more for higher frequencies.

Example 4.  A low-pass filter can be implemented by averaging the present input with the previous output. The result is an IIR filter. Show the filter equation, impulse response, transfer function, pole-zero plot, an magnitude of the FT. If the sampling rate is 200 Hz, what is the 3-dB cutoff frequency.

The filter equation is yn=xn+yn−12. This equation is a recursion and can be expanded as follows: yn.=.12xn+yn−1.=.12xn+12xn−1+yn−2.=.

12xn+12xn−1+yn−2.=..

12xn+14xn−1+14xn−2+yn−3

The impulse response has infinite number of terms:

hn.=.12δn+14δn−1+18δn−2+...

Take ZT of the filter equation: Yz=Xz+z−1Yz2. We have

Hz.=.YzXz.=.12−z−1.=.z−02z−12. The first-oder system  has ones zero at the origin and one pole at 1/2, as shown.

The FT is obtained by replacing z=ejω:

Hejω.=.12−cosω+jsinω.

The magnitude of the FT is:

Hejω.=.12−cosω2+sin2ω.

To find the 3-dB cutoff frequency, let

12−cosω2+sin2ω.=.12.

4−4cosω+cos2ω+sin2ω.=.2    ⇒

3−4cosω=.0    ⇒

ωc.=.cos−134.=.0.7227radians/s =

41.4o /s.

For a sampling rate of 200 Hz, this corresponds to fc=23Hz.

Compared to the 2nd-order FIR low-pass filter that has a cutoff frequency of 36 Hz, this filter has a lower cutoff frequency and a fast roll-off. However, unlike the FIR filters that have linear phase, this FIR filter has a nonlinear phase and may introduce phase distortion.

Example 5.  The above filter has a gain on 1/3 at the Nyquist frequency, which is half the sampling rate or  at ω=π.  Redesign the filter such that the gain is 0 at ω=π  .

The filter design can be done with the pole-zero plot. If the zero from the previous example is moved to -1, the gain should be brought down to 0 at ω=π without any significant changes on  other properties of the filter.

The new pole-zero plot is shown on the right.

The transfer function is

Hz.=.YzXz.=.14z+1z−12.=.z+14z−2.=.141+z−11−12z−1

Yz4−2z−1.=.Xz1+z−1    ⇒   Yz.=.14Xz+z−1Xz+2z−1Yz.

The filter equation in the time domain is:

yn.=.14xn+xn−1+2yn−1.

The FT is:

Hejω.=.121+e−jω2−e−jω.

The magnitude (as shown) is:

Hejω.=.121+cosω2+sin2ω2−cosω2+sin2ω.

To find the 3-dB cutoff frequency, let

121+cosω2+sin2ω2−cosω2+sin2ω.=.12    ⇒   1+cosω2+sin2ω.=.22−cosω2+sin2ω  ⇒

1+2cosω+cos2ω+sin2ω.=.8−8cosω+2cos2ω+2sin2ω    ⇒

10cosω.=.8    ⇒   ωc.=.cos−145.=.0.6435 radians/s =  36.9o/s.

For a sampling rate of 200 Hz, this corresponds to fc=20.5Hz.

Example 6.  A 60-Hz notch filter is commonly used for removing the 60-Hz line noise from the signal.  Design a FIR 60-Hz notch filter.

The filter design can be done with the help of the pole-zero plot. If the sampling rate is set at 240 Hz, π corresponds to 120 Hz and  π/2 corresponds to 60 Hz. Thus, a zero placed at π/2 will sink the 60 Hz noise. Another zero needs to be placed at  -π/2 in order to ensure that the coefficients of the filter equation are real number. Thus, we have a second-order system. The two poles are placed at the origin such that the filter is a FIR filter.

The transfer function is: Hz.=.12z−jz+jz−02.=.121+z−2.

2Yz.=.Xz+z−2Xz   ⇒  Yz.=.12Xz+z−2Xz

Filter equation: yn.=.12xn+xn−2;  Impulse response: hn.=.12δn+δn−2.

For a sampling rate of 240Hz, there are 4 sample points on a 60 Hz sinusoidal wave. Every other point has the opposite sign as shown. Thus, the averaging removes the 60 Hz wave .

The FT is: Hejω.=.1+e−2jω2.=.e−jωejω+e−jω2.=.e−jωcosω

The magnitude is: Hejω.=.cosω, as shown.

Example 7.  Combine the 60-Hz notch filter (example 6) with the 3-pint low-pass filter (example 2) with a cascade configuration.

When two LTI systems are connected in cascade. The overall impulse response if the convolution of the two individual impulse responses.

As shown in the figure, y1n=x1naah1n and

y2n=y1naah2n.

Combining the two, we have

y2n=x1naah1naah2n. Because the

convolution satisfies the associative property, we have

y2n=x1naah1naah2n=x1naahn.

Thus, hn=h1naah2n.

Low-pass filter:   h1n.=.14δn+2δn−1+δn−2

60 Hz notch filter:  h2n.=.12δn+δn−2

Combined: h1n.=.18δn+2δn−1+δn−2+δn−2+2δn−3+δ4.=.

18δn+2δn−1+2δn−2+2δn−3+δn−4

Figure on the right shows how this can be done graphically.

Because convolution in the time domain is equivalent to multiplication in the frequency domain, the  overall transfer function is the product of the individual transfer functions.

Hz.=.141+2z−1+z−2121+z−2.=.181+2z−1+2z−2+2z−3+z−4.

The filter equation is: yn.=.18xn+2xn−1+2xn−2+2xn−3+xn−4.

The FT is: Hejω.=.e−jωcosωe−jω21+cosω.=.e−2jωcosω1+cosω2.

The magnitude of Hejω is given by:

Hejω.=.cosω1+cosω2.

Example 8.  Analyze the following filter: yn.=.xn−xn−1+14yn−2.

Transfer function:  Take ZT, we have

Yz.=.Xz−z−1Xz+14z−2Yz⇒ Hz=YzXz.=.1−z−11−14z−2.=.1−z−11−12z−11+12z−1.

Impulse response:  Impulse response can be obtained by solving the recursion or by doing a long division as shown. From the long division, we have

hn.=.14n2aaaan:even−14n−12aan:odd

Frequency response:  Obtain the FT by substituting z=ejω. We have

Hejω.=.1−e−jω1−14e−2jω.=.2je−jω2ejω−e−jω2j34+12je−jωejω−e−jω2j.=.je−jωsinω232+je−jωsinω

Hejω.=.sinω2322+sin2ω, which is plotted on the right using the online graphing calculator at <www.desmos.com/calculator>.

Pole-zero plot:  The transfer function can be written as:

Hz.=.1−z−11−12z−11+12z−1.=.

z−0z−1z−12z+12

The system has two poles and two zeros at:

Poles: 12, −12

Zeros:  0,  1

The pole-zero plot is shown on the right.